2. Computer Architecture & Organization (CAO)

- Q: What is the primary difference between Von Neumann and Harvard architecture?

  - A: Von Neumann uses a single bus for data and instructions; Harvard uses separate buses.

- Q: What is "Pipelining"?

  - A: An implementation technique where multiple instructions are overlapped in execution.

- Q: Define "Cache Locality" (Temporal vs. Spatial).

  - A: Temporal: Recently accessed items likely to be accessed again. Spatial: Items near recently accessed items likely to be accessed.

- Q: What does "CISC" and "RISC" stand for?

  - A: Complex Instruction Set Computer and Reduced Instruction Set Computer.

- Q: What is the role of the Program Counter (PC)?

  - A: It holds the address of the next instruction to be fetched from memory.

- Q: What is "Microprogramming"?

  - A: A technique that uses a sequence of microinstructions to implement machine-code instructions.

- Q: Explain the purpose of a Translation Lookaside Buffer (TLB).

  - A: A hardware cache used to reduce the time taken to access a user memory location (virtual-to-physical mapping).

- Q: What is "DMA" (Direct Memory Access)?

  - A: A feature allowing hardware subsystems to access main memory independently of the CPU.

- Q: What is a "Wait State"?

  - A: A delay experienced by a processor when accessing slow memory or I/O devices.

- Q: What determines the "Word Size" of a computer?

  - A: The number of bits the CPU can process at one time (e.g., 32-bit or 64-bit).